IoT, Wireless Communication, and RTOS:- Introduction to IoT, IoT Architecture, Protocols (MQTT, CoAP), IoT Security Principles and Common Threats Wireless Communication: Interfacing GSM(Call, SMS, Internet), Bluetooth Communication Basics, LoRa Communication Basics and Applications, Designing an IoT-Based Home Automation System, Introduction to RTOS Concepts, Free RTOS with STM32: Task Creation, Scheduling, and Management, RTOS Timers, Delays, and RTC Integration,Inter-task Communication: Queues and Semaphores

Trust Zone Technology: Introduction to ARM Trust Zone, Trust Zone Architecture and Features, Secure and Non-Secure Worlds: Configuration and Management, Implementing Trust Zone in STM32U575, Advanced Debugging and Optimization: Code and Memory Optimization Techniques, Debugging Strategies and Tools.

## Introduction to IoT

IoT (Internet of Things) is a technology in which physical devices are connected to the Internet so that they can collect, exchange, and process data.

These devices can contain sensors, software, and communication systems that allow them to interact with other devices or systems.

### Examples of IoT

- 🏠 Smart home: Smart bulbs, fans, and thermostats can be controlled through a phone.

- ⌚ Smart watch: Measures heart rate, steps, sleep, etc., and sends the data to an app.

- 🚗 Connected cars: Can collect information about the vehicle and communicate with other systems.

- 🌾 Smart agriculture: Sensors can monitor soil moisture and help control irrigation.

### Main features of IoT

- Connectivity – Devices connect through the Internet or other networks.

- Sensing – Sensors collect information from the surroundings.

- Data processing – The collected data is processed to produce useful information.

- Automation – Devices can perform actions automatically based on the data.

- Communication – Devices can exchange information with other devices.

# IoT Architecture

IoT architecture describes the different layers through which data moves from a physical device to an application.

A commonly taught IoT architecture has four layers:

### 1. Perception Layer/Sensing Layer

- The lowest layer.

- Contains sensors and actuators.

- Sensors collect data from the physical environment.

- Actuators perform actions based on instructions.

Example: A temperature sensor measures room temperature.

### 2. Network Layer

- Transfers the collected data from devices to processing systems.

- Uses communication technologies such as Wi-Fi, Bluetooth, 4G/5G, Zigbee, etc.

Example: A smart thermometer sends its temperature reading through Wi-Fi.

### 3. Processing / Middleware Layer

- Receives and processes the collected data.

- May use cloud computing, databases, and data-processing systems.

- Converts raw data into useful information.

Example: A server analyses temperature readings and determines whether the room is too hot.

### 4. Application Layer

- Provides services to the end user.

- Shows the processed information or uses it to control devices.

Example: A mobile app displays the room temperature and allows the user to control the air conditioner.

### Simple flow

Physical World → Sensors → Network → Data Processing → Application/User

Example:
🌡️ Temperature sensor → 📶 Wi-Fi → ☁️ Cloud/server → 📱 Mobile app

## MQTT and CoAP Protocols

Protocols are rules that allow IoT devices to communicate and exchange data with each other.

### 1. MQTT — Message Queuing Telemetry Transport

MQTT is a lightweight messaging protocol designed mainly for IoT devices with limited resources and unreliable networks.

It uses a publish-subscribe model.

- Publisher: Sends data.

- Subscriber: Receives data.

- Broker: Acts as the middleman between publishers and subscribers.

- Topic: Identifies the type/category of data.

  Client: Any device (like a sensor or phone) that runs an MQTT library and connects to a broker to publish or receive data.

  Broker: The central server that receives messages from publishers and routes them to the correct subscribers

Example:
A temperature sensor publishes temperature data to the topic home/temperature. A mobile app subscribed to that topic receives the data.

Advantages:

- Lightweight and requires little bandwidth.

- Works well with unreliable networks.

- Supports communication between many devices.

- Suitable for devices with limited processing power.

### 2. CoAP — Constrained Application Protocol

CoAP is a lightweight web transfer protocol designed for resource-constrained IoT devices and networks.

It follows a request-response model, similar to HTTP.

- Client: Sends a request.

- Server: Processes the request and sends a response.

- Uses methods such as GET, POST, PUT, and DELETE.

Example:
A smart-home app sends a GET request to a temperature sensor, and the sensor responds with the current temperature.

Advantages:

- Lightweight and efficient.

- Requires less bandwidth than HTTP.

- Designed for devices with limited memory and processing power.

- Supports low-power networks.

### MQTT vs CoAP

| MQTT | CoAP |
| --- | --- |
| Publish-subscribe model | Request-response model |
| Uses a broker | Usually direct client-server communication |
| Commonly uses TCP | Commonly uses UDP |
| Good for continuous data/messages | Good for accessing individual IoT resources |
| Example: sensor continuously publishes readings | Example: app requests the current reading |

MQTT → Publish → Broker → Subscribe
CoAP → Request → Response

# IoT Security Principles & Common Threats

IoT security refers to protecting IoT devices, networks, data, and services from unauthorized access, attacks, damage, and misuse.

IoT Security Principles

### 1. Confidentiality

Ensures that only authorized users can access data.

Example: Encrypting data sent by a smart health device.

### 2. Integrity

Ensures that data is not modified or corrupted without authorization.

Example: An attacker should not be able to change sensor readings.

### 3. Availability

Ensures that IoT devices and services are available when required.

Example: A smart security system should continue functioning during an attack.

These three are called the CIA Triad:

C–Confidentiality
I–Integrity
A – Availability

### 4. Authentication

Verifies the identity of a user or device.

Example: A smart lock verifies an authorized user's credentials.

### 5. Authorization

Determines what an authenticated user/device is allowed to do.

Example: A user may view a camera but may not change its settings.

### 6. Non-repudiation

Ensures that a party cannot deny an action or transaction that it performed.

### 7. Privacy

Protects the personal and sensitive information collected by IoT devices.

Common Threats

### 1. Unauthorized Access

Attackers gain access to devices or systems without permission.

Example: Using weak/default passwords to access a smart camera.

### 2. Malware

Malicious software infects IoT devices and may allow attackers to control them.

### 3. Denial of Service (DoS)

An attacker sends excessive requests to make a device or service unavailable.

DDoS uses multiple compromised devices to perform the attack.

### 4. Eavesdropping

An attacker secretly intercepts communication between IoT devices.

### 5. Man-in-the-Middle (MITM)

An attacker secretly places themselves between two communicating devices and may intercept or alter the communication.

### 6. Data Theft

Sensitive information collected by IoT devices is stolen.

### 7. Spoofing

An attacker pretends to be a legitimate device or user.

### 8. Physical Attacks

An attacker physically accesses an IoT device to extract data, modify it, or damage it.

### 9. Firmware Attacks

Attackers exploit vulnerabilities in the device's firmware or install malicious firmware.

### 10. Weak Passwords

Default or easily guessable passwords can allow unauthorized access.

# Wireless Communication in IoT

Wireless communication allows IoT devices to communicate without physical cables.

Common wireless technologies include:

- Wi-Fi – high data rate, relatively higher power consumption

- Bluetooth/BLE – short range and low power

- Zigbee – low power, suitable for sensor networks

- GSM/Cellular – wide-area communication

- LoRa/LoRaWAN – long range and low power

# GSM Interfacing

GSM = Global System for Mobile Communications

# GSM Interfacing for Call

GSM (Global System for Mobile Communications) modules such as SIM800/SIM900 can be interfaced with a microcontroller to make and receive voice calls over a cellular network.

### Block Diagram

Microcontroller

|

UART/Serial

|

GSM Module

(SIM800/SIM900)

|

SIM Card

|

GSM Cellular

Network

|

Mobile Phone

### Components Required

- Microcontroller

- GSM module

- SIM card

- Antenna

- Power supply

- Mobile phone

### Working

- Insert an active SIM card into the GSM module.

- The microcontroller communicates with the GSM module through UART/serial communication.

- AT commands are sent from the microcontroller to control the GSM module.

- The command ATD<number>; is used to initiate a voice call.

- The GSM network establishes the call with the destination number.

- The command ATH is used to terminate the call.

### Important AT Commands

| AT Command | Function |
| --- | --- |
| AT | Checks communication with GSM module |
| ATD<number>; | Makes a voice call |
| ATA | Answers an incoming call |
| ATH | Terminates the call |

### Applications

- Emergency alert systems

- Security systems

- Remote monitoring

- Industrial automation

- IoT alarm systems

# GSM Interfacing for SMS

A GSM module can be interfaced with a microcontroller to send and receive SMS messages through the cellular network.

### Block Diagram

Sensor

↓

Microcontroller

↓ UART

GSM Module

↓

SIM Card

↓

Cellular Network

↓

Mobile Phone

### Working

- The microcontroller collects data from sensors or detects an event.

- It communicates with the GSM module through UART.

- The SMS mode is selected using the AT command AT+CMGF=1.

- The destination number is specified using AT+CMGS.

- The message is sent through the GSM cellular network.

- The receiving mobile phone receives the SMS.

### Important AT Commands

| AT Command | Function |
| --- | --- |
| AT | Checks GSM communication |
| AT+CMGF=1 | Selects SMS text mode |
| AT+CMGS | Sends SMS |
| AT+CMGR | Reads SMS |
| AT+CMGD | Deletes SMS |

### Example Sequence

AT

AT+CMGF=1

AT+CMGS="9876543210"

> Sensor Alert

> Ctrl+Z

The GSM module sends the specified message to the destination number.

### Applications

- Security alerts

- Fire/gas detection

- Health monitoring

- Industrial fault notification

- Smart agriculture

# GSM Interfacing for Internet

A GSM/GPRS module can provide Internet connectivity to a microcontroller or IoT device through a cellular network. It is useful when Wi-Fi is unavailable.

### Block Diagram

Sensors

↓

Microcontroller

↓ UART

GSM/GPRS Module

↓

SIM Card

↓

Cellular Network

↓

Internet

↓

Cloud / IoT Server

### Components Required

- Microcontroller

- GSM/GPRS module

- SIM card with data service

- Antenna

- Power supply

- Sensors/IoT devices

### Working

- The SIM card is inserted into the GSM/GPRS module.

- The module registers with the cellular network.

- The microcontroller communicates with the module through UART.

- AT commands are used to configure the cellular data connection.

- The GSM/GPRS module establishes a data connection through the mobile network.

- Sensor data can then be transmitted to an Internet/cloud server.

### Important AT Commands

Common commands include:

| AT Command | Function |
| --- | --- |
| AT | Checks communication |
| AT+CPIN? | Checks SIM status |
| AT+CSQ | Checks signal strength |
| AT+CREG? | Checks network registration |
| AT+CGATT? | Checks packet-domain attachment |
| AT+CSTT | Configures APN |
| AT+CIICR | Establishes wireless data connection |
| AT+CIFSR | Obtains IP address |

Exact commands vary depending on the GSM/GPRS module and firmware.

### Applications

- Remote IoT monitoring

- Smart agriculture

- Vehicle tracking

- Remote industrial monitoring

- Environmental monitoring

- Cloud-based sensor systems

### Advantages

- Internet connectivity over cellular networks

- Useful in remote locations

- Does not require local Wi-Fi

- Suitable for mobile and remote IoT devices

# Bluetooth Communication Basics

Bluetooth is a short-range wireless communication technology used to exchange data between electronic devices without cables. It operates mainly in the 2.4 GHz ISM (Industrial, Scientific and Medical) band.

### Main Features

- Short-range communication:
Typically used for communication over a few metres to tens of metres, depending on the Bluetooth class and version.

- Low power consumption:
Bluetooth Low Energy (BLE) is designed for very low-power IoT applications.

- 2.4 GHz frequency:
Bluetooth operates in the unlicensed 2.4 GHz ISM band.

- Wireless communication:
Devices communicate without physical cables.

- Data security:
Pairing, authentication and encryption help protect communication.

### Basic Bluetooth Communication

Bluetooth Device A → 2.4 GHz Radio Link → Bluetooth Device B

For example:

Sensor → Bluetooth Module → Smartphone

The sensor collects data and sends it through a Bluetooth module. A nearby smartphone or other Bluetooth-enabled device receives and processes the data.

### Bluetooth Network

Bluetooth devices can form a small network called a piconet. A piconet consists of a central device and one or more connected devices.

- Central: Manages the connection and communication.

- Peripheral: Connects to the central and provides or receives data.

In modern Bluetooth Low Energy (BLE), this central-peripheral model is commonly used.

### Bluetooth Communication Process

- Discovery: Devices identify nearby Bluetooth devices.

- Pairing: Devices establish a trusted connection.

- Connection: A communication link is established.

- Data Transfer: Data is exchanged between the connected devices.

- Disconnection: The connection is terminated when communication is no longer required.

### Bluetooth in IoT

Bluetooth, especially BLE, is widely used in IoT because of its low power consumption and support for short-range wireless communication.

Applications:

- Wearable devices

- Health-monitoring sensors

- Smart home devices

- Wireless keyboards and mice

- Beacons and asset tracking

- Mobile-to-device communication

### Advantages

- Low power consumption

- Low cost

- Easy to use

- No cables required

- Suitable for short-range IoT communication

- BLE provides long battery life

### Limitations

- Limited communication range

- Lower data rate than Wi-Fi

- Not suitable for long-distance communication

- 2.4 GHz interference can affect communication

LoRa

### Definition

LoRa (Long Range) is a low-power, long-range wireless communication technology mainly used in IoT applications. It is designed for transmitting small amounts of data over long distances with low power consumption.

### Main Features

- Long-range communication

- Low power consumption, suitable for battery-operated devices

- Low data rate

- Uses Chirp Spread Spectrum (CSS) modulation

- Suitable for large-scale IoT networks

- Good resistance to interference

- Suitable for devices that transmit data periodically

- Operates in sub-GHz ISM frequency bands, depending on the region

### LoRa Communication

A typical LoRa-based IoT system consists of end devices, gateways, network server and application server.

1. End Device[sensor nodes]:
Contains sensors and a LoRa communication module. It collects and transmits sensor data.

2. LoRa Gateway:
Receives LoRa radio signals from multiple end devices and forwards the data to the network server.

3. Network Server:
Manages the network, handles communication with gateways and processes device data.

4. Application Server:
Processes the data for a particular application and presents useful information to the user.

### LoRa and LoRaWAN

- LoRa is the physical-layer radio technology used for long-range communication.

- LoRaWAN is a network protocol and architecture that operates over LoRa and manages communication between end devices, gateways and servers.

### Advantages

- Long communication range

- Very low power consumption

- Long battery life

- Suitable for remote and battery-powered sensors

- Supports large-scale IoT deployments

- Low infrastructure and operating requirements

### Limitations

- Low data rate

- Not suitable for video or other high-bandwidth applications

- Limited payload size

- Not suitable where very low latency or continuous high-speed communication is required

# Applications of LoRa

### 1. Smart Agriculture

Sensors measure:

- Soil moisture

- Temperature

- Humidity

- Water level

The data can be sent over long distances to a central system.

### 2. Smart Cities

Used for:

- Smart street lighting

- Waste-bin monitoring

- Parking monitoring

- Environmental monitoring

### 3. Smart Metering

Used to remotely monitor:

- Electricity consumption

- Water consumption

- Gas consumption

### 4. Environmental Monitoring

Sensors can monitor:

- Air quality

- Temperature

- Humidity

- Flood/water levels

### 5. Industrial IoT

Used for:

- Machine monitoring

- Equipment condition monitoring

- Remote sensing

### 6. Asset Tracking

LoRa can be used to track the location/status of assets over large areas.

Here’s a KTU B.Tech 6-mark, exam-ready answer with the important points and a simple diagram.

# Designing an IoT-Based Home Automation System

An IoT-based home automation system is a system in which household appliances such as lights, fans, ACs and security devices are monitored and controlled automatically through the Internet.

### Basic Block Diagram

Sensors → IoT Controller → Internet/Wi-Fi → Cloud/Mobile App → User

User/Mobile App → Internet/Wi-Fi → IoT Controller → Relay/Actuator → Home Appliances

### Main Components

- Sensors:
Sensors such as temperature, light, motion and gas sensors collect information from the home environment.

- IoT Controller:
A microcontroller such as ESP32, ESP8266 or Arduino processes sensor data and controls appliances.

- Communication Network:
Wi-Fi, Bluetooth, Zigbee or other wireless technologies are used to communicate between the controller, Internet and user device.

- Cloud/IoT Platform:
Sensor data can be stored and processed on a cloud server. It also allows remote monitoring and control.

- Actuators:
Relays, motors and smart switches are used to control appliances such as lights, fans and doors.

- User Interface:
A mobile application or web interface allows the user to monitor sensor values and control appliances remotely.

### Working

- Sensors continuously collect information from the home.

- The IoT controller reads and processes the sensor data.

- The controller sends data to the cloud/mobile application through the Internet.

- The user can monitor the home and send control commands through the application.

- The controller receives the command and operates the required appliance through a relay or actuator.

- Automation rules can also be programmed, such as turning ON a light when motion is detected.

### Applications

- Smart lighting and fan control

- Temperature and AC control

- Security and intrusion detection

- Gas/smoke monitoring

- Energy monitoring

- Remote appliance control

### Advantages

- Remote monitoring and control

- Reduced energy consumption

- Improved convenience and safety

- Automatic operation of appliances

- Real-time monitoring

Yep — this is a 6-mark-type KTU question, and there are two parts. You should know the RTOS concepts plus the basic FreeRTOS task functions and scheduling.

## Introduction to RTOS

RTOS (Real-Time Operating System) is an operating system designed to execute tasks within a specified time limit. It is commonly used in embedded and IoT systems where timely response is important.

### Main RTOS Concepts

- Task:
A task is an independent unit of execution that performs a specific function.

- Scheduler:
The scheduler decides which task should run at a particular time.

- Task Priority:
Each task can be assigned a priority. Higher-priority tasks generally get CPU time before lower-priority tasks.

- Context Switching:
The CPU saves the state of one task and switches to another task.

- Task States:
A task can be in states such as Running, Ready, Blocked and Suspended.

### Advantages of RTOS

- Fast and predictable response

- Efficient task management

- Supports multitasking

- Suitable for time-critical embedded applications

- Better resource management

# FreeRTOS with STM32

FreeRTOS is a lightweight real-time operating system kernel commonly used with microcontrollers such as STM32.

It allows multiple tasks to run and manages CPU time using a scheduler.

### Task Creation

A task can be created using:

xTaskCreate();

General form:

xTaskCreate(TaskFunction,

"TaskName",

StackSize,

Parameters,

Priority,

&TaskHandle);

The task function contains the operations performed by the task.

Example:

void Task1(void *argument)

{

while(1)

{

// Task operations

vTaskDelay(1000);

}

}

### Important Task Management Functions

| Function | Purpose |
| --- | --- |
| xTaskCreate() | Creates a task |
| vTaskDelete() | Deletes a task |
| vTaskSuspend() | Suspends a task |
| vTaskResume() | Resumes a suspended task |
| vTaskDelay() | Blocks the task for a specified time |
| vTaskPrioritySet() | Changes task priority |

## Scheduling in FreeRTOS

The FreeRTOS scheduler decides which ready task gets the CPU.

- Tasks are assigned different priorities.

- A higher-priority ready task can run before a lower-priority task.

- In preemptive scheduling, a running lower-priority task can be interrupted when a higher-priority task becomes ready.

- vTaskDelay() temporarily blocks a task, allowing other ready tasks to execute.

### Example

Suppose there are three tasks:

- Task 1: Sensor reading – Priority 3

- Task 2: Display update – Priority 2

- Task 3: LED control – Priority 1

If all three are ready, the scheduler gives preference to Task 1 because it has the highest priority.

# RTOS Timers, Delays, and RTC Integration

An RTOS (Real-Time Operating System) provides timing mechanisms that allow tasks to execute at specified times or intervals. Timers, delays and Real-Time Clocks (RTC) are commonly used in embedded and IoT systems.

### 1. RTOS Timers

An RTOS timer is a software mechanism used to perform an action after a specified time or at regular intervals.

Types of RTOS timers:

- One-shot timer: Executes its action only once after the specified time.

- Auto-reload timer: Repeatedly executes its action at fixed intervals.

Applications:

- Periodic sensor readings

- LED blinking

- Communication timeouts

- Periodic data transmission

### 2. Task Delays

A task delay temporarily blocks a task for a specified period. During this time, the processor can execute other ready tasks.

In FreeRTOS, the function:

vTaskDelay()

is commonly used to delay a task.

Example:

vTaskDelay(pdMS_TO_TICKS(1000));

This delays the task for approximately 1000 ms.

Advantages:

- Allows other tasks to execute

- Reduces unnecessary CPU usage

- Useful for periodic operations

### 3. RTC (Real-Time Clock)

An RTC is a hardware or dedicated clock module that maintains the actual date and time, even when the main processor is performing other operations.

It is used for:

- Date and time stamping

- Data logging

- Alarms

- Scheduled operations

- Time-based automation

### 4. RTC and RTOS Integration

The RTC can provide actual calendar time while the RTOS handles task scheduling and software timing.

Example:

RTC → Provides current date/time
RTOS → Schedules tasks
Task → Reads sensor → Adds timestamp → Stores/transmits data

For example, an IoT data logger can read temperature every 10 seconds using an RTOS timer and store the measurement with the current date and time obtained from the RTC.

### Difference Between RTOS Timer and RTC

| RTOS Timer | RTC |
| --- | --- |
| Software-based timing mechanism | Hardware/dedicated clock |
| Used for task timing | Maintains actual date and time |
| Used for periodic operations | Used for timestamps and schedules |
| Does not represent calendar time | Maintains calendar time |

# Inter-task Communication: Queues and Semaphores

Inter-task communication (ITC) is the mechanism used by tasks in an RTOS to exchange data and synchronize their activities. Queues and semaphores are two commonly used mechanisms in FreeRTOS.

### 1. Queues

A queue is a kernel-managed data structure used to transfer data between tasks. It normally follows the FIFO (First In, First Out) principle.

Basic operation:

Producer Task → Queue → Consumer Task

The producer places data into the queue, and the consumer retrieves the data.

Important FreeRTOS functions include:

- xQueueCreate() – creates a queue

- xQueueSend() – sends data to a queue

- xQueueReceive() – receives data from a queue

- vQueueDelete() – deletes a queue

Applications:

- Sending sensor data between tasks

- Communication between producer and consumer tasks

- Passing messages between tasks

- Buffering data

### 2. Semaphores

A semaphore is an RTOS synchronization mechanism used to coordinate tasks and control access to resources.

The two common types are:

Binary Semaphore:
Has two states: available and taken. It is commonly used for task synchronization or signalling.

Counting Semaphore:
Maintains a count and can be used when multiple instances of a resource are available.

### Semaphore Operation

Task/Event → Semaphore → Waiting Task

For example, an interrupt can give a semaphore when data arrives. A waiting task receives the semaphore and starts processing the data.

Common FreeRTOS functions include:

- xSemaphoreCreateBinary() – creates a binary semaphore

- xSemaphoreTake() – takes/waits for a semaphore

- xSemaphoreGive() – gives/releases a semaphore

### Queue vs Semaphore

| Queue | Semaphore |
| --- | --- |
| Used mainly for data transfer | Used mainly for synchronization |
| Can store multiple data items | Usually represents a signal/resource |
| Generally follows FIFO | Indicates availability/event |
| Producer sends data | One task/event signals another |

### Example

Consider an IoT temperature monitoring system:

Sensor Task → Queue → Processing Task

The Sensor Task sends temperature readings through a queue. A semaphore can be used to signal the Processing Task whenever new sensor data is ready.

Here’s a KTU B.Tech 6-mark, exam-ready answer covering both the introduction and architecture/features.

# Introduction to ARM TrustZone, Architecture and Features

ARM TrustZone is a hardware-based security technology developed by ARM to protect sensitive data and applications in embedded systems and IoT devices. It divides the processor and system resources into two isolated environments: Secure World and Normal World.

### Basic Architecture

Application / OS
↓
Normal World ←→ Secure Monitor / Secure Gateway ←→ Secure World
↓ ↓
Normal Resources Secure Resources

### 1. Secure World

The Secure World is used for security-sensitive operations and resources.

Examples:

- Encryption and decryption

- Secure key storage

- Authentication

- Secure boot

- Trusted applications

Only trusted software is allowed to access protected secure resources.

### 2. Normal World

The Normal World runs normal applications and operating systems such as Linux or Android.

It has access to non-sensitive resources but cannot directly access protected Secure World resources.

### 3. Secure Monitor / Gateway

A controlled mechanism is provided for communication between the Normal and Secure Worlds.

When the Normal World needs a protected service, it makes a controlled request to the Secure World. The Secure World performs the operation and returns the result.

### Important Features of TrustZone

- Hardware-based isolation:
Secure and normal resources are separated using hardware-supported mechanisms.

- Secure and Normal Worlds:
Provides two execution environments with different security levels.

- Secure memory and peripherals:
Sensitive memory regions and peripherals can be protected from normal software.

- Secure boot:
Helps ensure that only trusted software is executed during system startup.

- Protection of cryptographic keys:
Sensitive keys and security credentials can be stored and processed in the Secure World.

- Controlled communication:
Secure services can be accessed from the Normal World only through controlled interfaces.

### Applications

- IoT device security

- Secure payment systems

- Digital rights management

- Secure authentication

- Automotive systems

- Smart devices and embedded systems

### Advantages

- Protects sensitive information

- Prevents normal applications from directly accessing secure resources

- Provides hardware-assisted security

- Helps protect cryptographic keys and trusted applications

- Suitable for resource-constrained embedded and IoT devices

# Secure and Non-Secure Worlds: Configuration and Management

ARM TrustZone divides a processor system into two security domains: Secure World and Non-Secure World. This separation protects security-critical resources from normal applications.

### 1. Secure World

The Secure World contains trusted software and security-sensitive resources.

Examples:

- Secure boot

- Cryptographic operations

- Encryption keys

- Authentication

- Trusted applications

- Security-critical peripherals

Only authorized secure software can access these resources.

### 2. Non-Secure World

The Non-Secure World runs normal applications and operating systems.

Examples:

- User applications

- IoT application software

- Communication protocols

- Normal peripheral operations

Non-Secure software cannot directly access resources assigned to the Secure World.

### 3. Configuration of Security Domains

Memory, peripherals and other system resources are assigned as Secure or Non-Secure during system configuration.

The processor uses hardware security attributes to determine whether an access is permitted.

Example:

Secure Memory → Secure Software only
Non-Secure Memory → Normal Software
Secure Peripheral → Secure Software only
Non-Secure Peripheral → Normal Software

### 4. Communication Between Worlds

The Non-Secure World can request services from the Secure World through a controlled Secure Gateway.

Non-Secure Application → Secure Gateway → Secure Function → Result

This prevents direct and unauthorized access to secure resources.

### 5. Management

TrustZone security is managed by:

- Assigning memory regions as Secure/Non-Secure

- Assigning peripherals to the appropriate security domain

- Controlling access permissions

- Protecting secure functions

- Managing transitions between Secure and Non-Secure execution

- Preventing unauthorized Non-Secure access

### Advantages

- Isolation of sensitive resources

- Protection against unauthorized access

- Secure execution of trusted applications

- Protection of cryptographic keys

- Improved security for IoT and embedded systems

# Implementing TrustZone in STM32U575

The STM32U575 is an STM32 microcontroller based on the Arm Cortex-M33 processor and supports Arm TrustZone technology. TrustZone can be used to divide the microcontroller system into Secure and Non-Secure execution environments.

### Basic Structure

STM32U575

→ Secure Application
→ Secure Memory / Secure Peripherals

↕ Secure Gateway

→ Non-Secure Application
→ Non-Secure Memory / Peripherals

### Steps for Implementation

- Enable TrustZone

TrustZone is enabled in the microcontroller configuration so that the processor can operate with Secure and Non-Secure states.

- Configure Memory

Flash and SRAM regions are divided into Secure and Non-Secure areas.

  - Secure region → Security-critical code/data

  - Non-Secure region → Normal application code/data

- Configure Peripherals

Peripherals are assigned to the appropriate security domain. Sensitive peripherals can be kept Secure, while normal peripherals can be made Non-Secure.

- Configure Security Attribution

Security attribution mechanisms are used to determine whether memory and peripherals are accessible from the Secure or Non-Secure state.

- Create Secure and Non-Secure Applications

The Secure application handles security-critical operations, while the Non-Secure application performs normal application functions.

- Use Secure Gateway

Non-Secure software accesses selected Secure functions through controlled Secure Gateway (SG) entry points rather than directly accessing Secure resources.

### Example

Consider an STM32U575 IoT device storing an encryption key:

Non-Secure Application
↓
Secure Gateway
↓
Secure Function
↓
Encryption Key in Secure Memory

The encryption key remains inside the Secure environment, while the Non-Secure application can request encryption through an authorized Secure function.

### Tools Used

TrustZone development for STM32U575 can be configured using STM32CubeMX/STM32CubeIDE and the STM32 security configuration tools.

### Benefits

- Protection of sensitive data and keys

- Isolation of security-critical code

- Secure execution of trusted functions

- Protection against unauthorized peripheral and memory access

- Suitable for secure IoT and embedded applications

Write a program to implement trust zone in STM32U575

### Objective

To demonstrate communication between the Secure World and Non-Secure World using a Secure Gateway function.

## 1. Secure World Program

The secure application contains a function that can be called by the Non-Secure application.

#include "main.h"

void Secure_LED_On(void)

{

HAL_GPIO_WritePin(GPIOA, GPIO_PIN_5, GPIO_PIN_SET);

}

The function can be exposed as a Non-Secure Callable (NSC) function using the TrustZone configuration.

## 2. Non-Secure World Program

The Non-Secure application calls the secure function through the Secure Gateway.

#include "main.h"

/* Secure function */

extern void Secure_LED_On(void);

int main(void)

{

HAL_Init();

/* System and GPIO initialization */

SystemClock_Config();

MX_GPIO_Init();

while (1)

{

/* Call Secure World function */

Secure_LED_On();

HAL_Delay(1000);

}

}

### Working

- The STM32U575 starts execution in the Secure World.

- Secure memory and peripherals are configured.

- The secure function Secure_LED_On() is made available through a Secure Gateway.

- The Non-Secure application calls the secure function.

- The Secure World executes the function.

- HAL_GPIO_WritePin() turns ON the LED.

- HAL_Delay() creates a 1-second delay.

### Block Diagram

NON-SECURE WORLD

|

| Secure Function Call

↓

Secure Gateway (SG)

|

↓

SECURE WORLD

|

↓

HAL_GPIO_WritePin()

|

↓

LED

### Important Points

- STM32U575 uses an Arm Cortex-M33 processor.

- TrustZone provides Secure and Non-Secure execution states.

- HAL functions such as HAL_GPIO_WritePin() can be used in the application.

- Secure functions must be exposed through an appropriate Non-Secure Callable (NSC) interface.

- Memory and peripheral security attributes are configured using STM32 security configuration tools/CubeMX.

- The Non-Secure application should not directly access resources assigned to the Secure World.

# Code and Memory Optimization Techniques

Optimization is the process of improving an embedded program so that it uses less memory and CPU time while maintaining the required functionality.

## 1. Code Optimization Techniques

### 1. Reduce unnecessary code

Remove unused functions, variables, libraries and duplicate code to reduce program size.

### 2. Optimize loops

Avoid unnecessary calculations inside frequently executed loops. Move constant calculations outside the loop whenever possible.

### 3. Use suitable data types

Use the smallest suitable data type for variables.

Example:

uint8_t count;

can be used instead of a larger integer type when the value is within its range.

### 4. Avoid unnecessary function calls

Frequently called small functions can be optimized using appropriate compiler optimization techniques such as inlining.

### 5. Use compiler optimization

Compilers such as GCC provide optimization levels such as:

-O0 → No optimization

-O1 → Basic optimization

-O2 → Higher optimization

-O3 → Aggressive optimization

-Os → Optimize for size

For embedded systems, -Os can be useful when program memory is limited.

## 2. Memory Optimization Techniques

### 1. Reduce RAM usage

Avoid unnecessarily large arrays and variables.

### 2. Use constant data efficiently

Constant data can be placed in program/Flash memory instead of consuming RAM where appropriate.

### 3. Optimize data structures

Arrange structures and select data types carefully to reduce unnecessary memory usage.

### 4. Avoid memory fragmentation

In embedded systems, excessive dynamic memory allocation using malloc() and free() can cause fragmentation. Static allocation or memory pools can be preferred for predictable systems.

### 5. Optimize stack usage

Avoid unnecessarily large local arrays and excessive recursion to prevent stack overflow.

## 3. Benefits

- Reduced Flash and RAM usage

- Faster execution

- Lower power consumption

- Better system responsiveness

- More efficient use of microcontroller resources

# Debugging Strategies and Tools

Debugging is the process of finding, identifying and correcting errors or faults in an embedded system.

## 1. Debugging Strategies

### 1. Identify the problem

First determine the symptoms and conditions under which the fault occurs.

### 2. Reproduce the error

Run the program repeatedly under the same conditions to identify when and how the error occurs.

### 3. Use breakpoints

A breakpoint temporarily stops program execution at a selected line so that variables and processor state can be examined.

### 4. Step-by-step execution

Execute the program one instruction or line at a time to find where the incorrect behaviour begins.

### 5. Check variables and registers

Monitor variable values, CPU registers, memory contents and peripheral registers during execution.

### 6. Use logging and trace information

UART/serial messages, logs and trace data can help identify program flow and runtime errors.

## 2. Common Debugging Tools

### 1. Debugger

An IDE debugger allows the programmer to:

- Set breakpoints

- Step through code

- Inspect variables

- Examine registers

- Watch memory

### 2. ST-LINK

ST-LINK is commonly used with STM32 microcontrollers for programming and debugging through interfaces such as SWD.

### 3. STM32CubeIDE

STM32CubeIDE provides code development, compilation and debugging facilities for STM32 devices.

### 4. Serial/UART Debugging

Debug messages can be sent through UART and viewed on a computer using a serial terminal.

### 5. Logic Analyzer

A logic analyzer is useful for examining digital communication signals such as UART, SPI and I²C.

### 6. Oscilloscope

An oscilloscope can be used to observe electrical signals, timing, frequency and signal integrity.

## 3. Typical Debugging Process

Detect Fault

↓

Reproduce Fault

↓

Set Breakpoints / Add Logs

↓

Examine Variables & Registers

↓

Identify Root Cause

↓

Modify Code

↓

Test Again

## 4. Common Embedded-System Errors

- Incorrect pointer or memory access

- Stack overflow

- Incorrect peripheral configuration

- Timing errors

- Interrupt-related problems

- Communication errors

- Incorrect variable values